Appendix D: Analysis of the influence of θY on the reconstructed shape

The principles of shape reconstruction, under the conditions of θY=0 and θY0, are shown in Figures 1(a) and 1(b).

Figure 1. Influence of posture angle Y on the reconstructed surface of the planar sample. The symbol  represents a planar region on the surface of the planar sample, which is parallel to the XOY plane and divided into L small sub-regions along the X-axis. These sub-regions are denoted as {Ul|l=1L}, or simply ={Ul}. {Ul′} represents the reconstructed surface of all elements in {Ul}, and ′ is used to describe the composite surface formed from {Ul′}. (a) Reconstruction process at Y =0; (b)  Reconstruction process at θY0.

In Figure 1(a), the field of view is the area enclosed by the four vertices A, B, C, and D, known as SABCD. Since the image sensor is a rectangular array, SABCD represents a rectangular plane area with its normal parallel to the optical axis. As the plane area  is translated along the X-axis, its local regions enter the field of view SABCD. Depending on the size of SABCD,  is divided into L rectangular sub-areas {Ul}, which are arranged along the X-axis and are the same size as SABCD. U1UL sequentially enters the field of view and fills SABCD. When they are in the field of view SABCD, they are denoted as {Ul. SABCD|l=1L}, and all units in {Ul. SABCD} completely coincide with SABCD. We reconstruct the units in {Ul. SABCD}, and the reconstruction process is outlined by the following formula.

Ul'.SA'B'C'D'|l=1→L=Recon({Ul.SABCD|l=1→L})                         (D_1)

Where Recon(·) represents the reconstruction process outlined in Sec. 2.3. Points A′, B′, C′, and D′ are the reconstructed points corresponding to vertices A, B, C, and D. Ul′. SA′B′C′D′ denotes the region generated after reconstructing Ul. SABCD. Since the units in {Ul. SABCD} perfectly coincide, and assuming there are no differences in the properties of the units (such as texture), all units in {Ul′. SA′B′C′D′} also perfectly coincide in O-XYZ. The next step is to translate and expand the units in {Ul′. SA′B′C′D′} along the X-axis and recover the result to form {Ul′}. {Ul′} is the reconstruction result of {Ul}, and this process is described by the following equation.

λ'=Ul'|l=1→L=Unfold.X({Ul'.SA'B'C'D'|l=1→L})                       (D_2)

Where Unfold.X(·) refers to the method of unfolding the surface along the X-axis, and ′ denotes the surface composed of {Ul′}. Ul is translated along the X-axis by a distance of (l-1)|AB| to obtain Ul. SABCD. Similarly, Ul′. SA′B′C′D′ is translated in the opposite direction along the X-axis by a distance of (l-1)|AB| to obtain Ul′. Since SABCD is parallel to the XOY plane, the image points of all object points in {Ul.SABCD} are located within the same image frame, meaning they share the same z-coordinate. According to Eq. (D_1), all reconstructed points in {Ul′. SA′B′C′D′} also possess the same Z-coordinate, indicating that all units in {Ul′. SA′B′C′D′} not only overlap but are also parallel to the XOY plane. The surface ′ generated by unfolding along the X-axis must be a planar area.

In Figure 1(b), U1UL also shifts along the X-axis into SABCD to generate {Ul. SABCD}, and subsequently, {Ul′. SA′B′C′D′} is obtained through the reconstruction process. Unlike in Figure 1(a), there is an angle θY between the normal vectors of SABCD and SEBCF. During image acquisition, object points in SABCD move along the Z-axis through the inclined SEBCF. The image points corresponding to these object points may be distributed across different image frames and possess varying z-coordinates. Consequently, the reconstructed points in {Ul′. SA′B′C′D′} may also have different Z-coordinates, and their normal vectors are no longer parallel to the Z-axis. Applying the method outlined in Eq. (D_2), {Ul′. SA′B′C′D′} is unfolded to generate {Ul′} and ′. The units in {Ul′} remain parallel, but their normal vectors are misaligned with the Z-axis, resulting in a wavy pattern. As a result, ′ is no longer a planar region but instead exhibits a wavy structure surface.